# Task 5: Neural-ODE model of the Silverbox

Code:

# 1. Data

The excerpt is samples [42650 : 46746] of SNLS80mV.mat (4096 samples, fs = 610.35 Hz, Δt = 1/fs). I subtracted the mean of u and the mean of y on this excerpt. The first 3072 samples are the training record and the last 1024 samples are the test record.

# 2. State equation

With velocity v = ẏ, the state is (y, v) and

ẏ = v,    v̇ = α_θ(y, v, u)

where α_θ is a fully connected network and u(t) is the measured input. I integrate this with classical Runge–Kutta 4 at step Δt. The input is known only at the sample times, so the half-step value is the average of its neighbours, u(k+½) = (u_k + u_(k+1)) / 2.

# 3. Architecture kept

The inputs are scaled so that the network sees numbers of order 1: (y/S_y, v/S_v, u/S_u). Here S_y and S_u are the standard deviations of y and u on the training record, S_v = ω0·S_y and ω0 = 2π·70. The output is multiplied by S_a = ω0²·S_y, because the accelerations are of order 10^4 V/s².

- Linear(3, 32) + tanh → Linear(32, 32) + tanh → Linear(32, 1)

- A parallel linear path Linear(3, 1), added to the output (it can represent a spring, a damper and an input gain)

- Output = (MLP + linear path) × S_a

- Number of parameters: 128 + 1056 + 33 + 4 = 1221

# 4. Training

Training has two stages. Both use only the training record (samples 0 to 3071).

Stage 1: acceleration matching. The target is the second central difference a_k = (y_(k+1) − 2y_k + y_(k−1)) / Δt². The inputs are y_k, a central-difference velocity, and u_k. The loss is the mean squared error divided by S_a². I used Adam with learning rate 3·10⁻³ and 3000 full-batch steps. Its purpose is to start the network near the right dynamics. The loss fell from 6.1·10⁻¹ to 1.1·10⁻³.

Stage 2: rollout fine-tuning. Each step uses a batch of 32 random segments of the training record. The state is initialized from the measured y and the central-difference velocity. It is integrated with RK4 for L steps, and the loss is the mean squared error of the predicted y against the measured y, divided by S_y². I used Adam with learning rate 10⁻³. The rollout length grows in three stages:

| Rollout length L | Steps |
| --- | --- |
| 16 | 300 |
| 64 | 200 |
| 128 | 100 |

# 5. Evaluation (free run)

I do one free run of the whole excerpt. It starts at sample 0 with y(0) and v(0) = (y_1 − y_0)/Δt. It is integrated through the 3072 training samples and on into the 1024 test samples, keeping the same state across the boundary. After the first two samples, the rollout reads only u and never y.

Kept version:

|  | Free-run RMSE |
| --- | --- |
| Train (3072 samples) | 0.00152 V |
| Test (1024 samples) | 0.00496 V |
| Baseline (predict zero), test | about 0.06 V |

The test RMSE is about 12 times smaller than the baseline. The test error is about 3 times the train error, a modest gap. I attribute part of it to the test segment coming after 3072 free-run steps, so the state error has accumulated, and part to the test record containing the largest peaks.

Plot. [Insert test_free_run.png here: measured against predicted output on the test segment, free run. In the submitted figure, remove the dashed least-squares line.]

# 6. Other settings tried

| # | Setting | Test RMSE (V) |
| --- | --- | --- |
| 1 | Next-step model y(k+1) = f(y_k, y_(k−1), u_k), 3×128 tanh, 400 steps, no ODE | 0.0164 |
| 2 | Acceleration net 2×32 + linear path, RK4 rollout training only (no pretraining), lr 3·10⁻³ | 0.0598 |
| 3 | Least-squares fit of equation (1) (not a neural network), same RK4 | 0.0299 |
| 4 | Kept: acceleration net 2×32 + linear path, pretraining + rollout fine-tuning | 0.0050 |
| 5 | [e.g. width 64, otherwise as #4] | [ ] |
| 6 | [e.g. 3 hidden layers] | [ ] |
| 7 | [e.g. SiLU instead of tanh] | [ ] |
| 8 | [e.g. last rollout stage (128, 300)] | [ ] |

- Row 2 vs row 4: rollout training from random initialization got stuck near the “predict zero” solution, so its test RMSE equals the baseline. Pretraining on finite-difference accelerations fixed this.

- Row 3: the fit gave a ≈ 39.6 s⁻¹, b ≈ 1.76·10⁵ s⁻², c ≈ 5.7·10⁵ and g ≈ 1.6·10⁵. These are consistent with the task’s description (damping of a few tens, b near (2π·70)² ≈ 1.93·10⁵). It also showed that the RK4 integrator itself reproduces the waveform, so the integrator was not the cause of row 2. Its train RMSE was 0.0222 V.